Project Report

| Submitted by : | Arsal Naveed |
| --- | --- |
| Submitted to : | Muhammad Fahad Irshad |
| Student id : | F2023266900 |
| Section : | V7 |

# Mini OS Simulator: Technical Report

## 1. Executive Summary

The Mini OS Simulator is an integrated C++ application designed to model the core functions of an operating system kernel. It simulates three critical subsystems: Process Scheduling, Concurrent Task Management (Producer-Consumer), and Deadlock Prevention (Banker’s Algorithm). By utilizing POSIX threads and synchronization primitives, the system provides a realistic environment for observing how a kernel manages resources and schedules tasks in real-time.

## 2. System Architecture & Modules

The project is built using a modular approach to ensure scalability and ease of debugging.

### A. Process & Scheduler Module

This module handles the execution of processes residing in the Ready Queue.

- Dynamic Algorithm Switching: The system evaluates the system load and chooses the most efficient algorithm:

  - Priority Scheduling (Non-preemptive): Used when the Ready Queue has 5 or fewer processes. This ensures that high-priority tasks are handled quickly in a low-load environment.

  - Round Robin (RR) Scheduling (Preemptive): Used when the queue exceeds 5 processes. It uses a Time Quantum of 3 units to prevent process starvation and ensure fair CPU time distribution.

- Performance Metrics: After execution, the module calculates and displays Waiting Time, Turnaround Time (TAT), and Average Statistics.

### B. Producer-Consumer Module

This module manages the entry of tasks into the system using a multithreaded architecture.

- Producers: Two independent threads generate processes with randomized attributes (Burst Time, Priority, and Resource Needs) and attempt to place them into a Bounded Buffer of size 5.

- Consumer: A single thread acts as the "Middleman." it retrieves processes from the buffer, subjects them to a safety check, and moves them to the appropriate queue (Ready or Blocked).

### C. Resource Management & Deadlock Prevention

This module acts as the system's "Safety Officer" using the Banker’s Algorithm.

- Pre-allocation Check: Before a process is allowed to enter the Ready Queue, the system simulates the allocation of its maximum requested resources.

- Safety Logic: If the system cannot guarantee a "Safe Sequence" where all processes can eventually finish, the requesting process is moved to a Blocked Queue to prevent a potential deadlock.

## 3. Synchronization & Thread Safety

Concurrency is managed through strictly defined synchronization primitives to prevent race conditions and busy-waiting.

| Primitive | Role in Simulator |
| --- | --- |
| empty_slots (Semaphore) | Tracks available space in the buffer. Producers wait here if the buffer is full. |
| full_slots (Semaphore) | Tracks occupied spaces in the buffer. The consumer waits here if the buffer is empty. |
| mutex_lock (Mutex) | Ensures that only one thread (Producer 1, Producer 2, or Consumer) can modify the buffer or queues at any given time. |

## 4. Implementation Details

### Data Structures

The core of the simulator is the Process struct, which tracks:

- Execution Data: PID, Burst Time, and Remaining Time.

- Resource Matrices: max_need, allocated, and need arrays (supporting 3 resource types: R0, R1, R2).

### Integrated Simulation Flow

- System Initialization: Resources are set (e.g., 10 units of R0, 5 of R1, 7 of R2).

- Concurrency Start: Worker threads (Producers/Consumer) begin operating in the background.

- Kernel Menu: The user interacts with the "Live" system to view the state, run the scheduler, or force-inject specific processes.

## 5. Limitations & Future Scope

### Current Limitations

- Non-Preemptive Priority: While Round Robin is preemptive, the Priority Scheduling implementation waits for a process to finish its entire burst.

- Static Resource Types: The number of resource types is hardcoded to 3.

- Memory Simulation: The simulator lacks a Virtual Memory/Paging module.

### Future Expansion (FYP Scope)

- Developing a GUI-based Dashboard using Qt or Dear ImGui.

- Implementing Cloud Workload Scheduling to simulate distributed resource management.

- Adding a Memory Management Unit (MMU) to simulate paging and segmentation.

## 6. Conclusion

The Mini OS Simulator successfully demonstrates the integration of complex OS concepts into a working, multithreaded system. By preventing deadlocks and dynamically adapting scheduling strategies, the project reflects the real-world logic used in modern operating system kernels.

Project Repository:  https://github.com/arsal626/Operating-Lab-Project.git